Table 1 TEA of Key ESS for modelling in 2035/2050

| Energy Storage Technologies | Cha efficiency % | Discha efficiency % | Self-loss rate %/day | RTE % | Cha power cost $/kW | Energy store cost $/kWh | Discha power cost $/kW | FOM $/kW.year | VOM $/MWh | LCOS $/kWh | Lifetime-years/ Mul-vector |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vanadium redox flow Batt (VRF) | 0.90 | 0.90 | 0.001 | 0.801 | 1771,2 | 3862 | 1771,2 | 11.32, | 0.51252 | 319 | 15, E |
| Compressed air ESS (CAES)a | 0.85 | 0.90 | 0 | 0.651 | 5452, | 3.43 | 6722 | 12.92 | 1.72 | 106 | 25, E/H |
| Liquid air ESS (LAES)b | 0.60 | 0.904 | 1, | 0.553 | 7244 | 68.85 | 8062,5 | 15.52,5 | 1.852,5 | 300 | 25, E/H |
| Rankine-PTES CO2 (LCES)c | 0.85 | 0.88 | 0.56 | 0.577 | 6428,9 | 98.53,9 | 7511,9 | 39.410 | 3.5 | 270 | 25, E/H |
| Brayton-PTES air (BPTES)d | 0.851 | 0.901 | 16 | 0.61 | 6341,10,5,11 | 87.83,10 | 6851,5,11 | 27.610 | 3.5 | 350 | 25, E/H |
| Hydrogen ESS (H2ES)e | 0.7312 | 0.5812 | 0.512 | 0.401,13 | 8101,2 | 3.92 | 9561,2 | 28.52, | 1.3 | 237 | 30, E/H/G |
| Sensible heat (H2O) | 0.98 | 0.984, | 0.05 | \ | \ | 5.74,15 25.3(ice)15 | \ | 30 | 0.01 | \ | 30, E/H |
| Sensible heat (Gravel) | 0.90, | 0.904, | 1 | \ | \ | 34.515 | \ | 30 | 0.01 | \ | 30, E/H |
| Latent heat-PCM ((molten salt) | 0.95, | 0.954, | 0 | \ | \ | 43.514,15 | \ | 120 | 0.1 | \ | 30, E/H |
| Thermochemical-TCM (CaO/ CaOH2) | 0.95, | 0.954, | 0 | \ | \ | 4115 | \ | 25 | 0.1 | \ | 25, E/H |

Notes:

- In the work13, the average cost of electricity fed into the grid by hydrogen storage ranges from £52/MWh to £92/MWh. OPEX of hydrogen storage and CAES are assumed as and 1.5% and 4% of CPAEX.

- CAES has a typical construction time of three years, 95% availability, and 99% reliability at starting time 16.

Table 3 TEA of Key ESS for modelling - 2030

| Energy Storage Technologies | Energy density (kWh/m3) | LCOS-power $/kW | LCOS-energy $/kWh | Rated charge hours/h | Rated discharge hours/h | Storage duration | Lifespan (years) | TRL Charge hours/h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compressed air energy storage5 | 3 - 15 0.5- 20 3 – 617, 2.918, | 1,047-1,481 970-1,200, 893-1315€/kW2, 1600-230019, 166919(2017), 13812, | 4-50 20-220, 40(above ground)-109(underground)2, 43-16219, 772, | 10-30 | 8-16 | 10-40 h19 8-2012, H - M | 20 - 40 | Diabatic Adiabatic Isothermal |
| Vanadium redox battery | 20-701, 35-60 Wh/kg8, 10-75 Wh/kg2, | 3994(2020)2, 3187(2030)2, | 399(2020)2, 319(2030)2, | 10 | 10 | H - D | 152, | TRL – 7~8 |
| Liquid air energy storage5 | 50-80, 667, | 900-6,000, 1270-2090€/kW, 700–30007, 9507, | 240-640, 400-8007, 200-5007, 1237, |  |  | H - M | 30 - 40 | LCOS 218 $/MWh |
| Rankine PTES (CO2 based) | 157, 12.7, 5020, | 550–7537, 500–80007, 750-150014，300-280020, | 275–3767, 250–10007, 0.226, 25-25020, |  |  | H - M | >25 | LCOS 230 or 180-440 $/MWh, TRL 4-714 |
| Resistive heat CB |  | 1300-150010, | 18610, |  |  | H - M | >25 | LCOS 186 $/MWh TRL 7-8 |
| Sensible heat | 15-80 25-85 70-200, 82(water), 25-12017, 10-120 Wh/kg17, 23.221 | 400-7003 |  |  |  | H - M W - M H - M H – D | 15-40 50 >5000 cycles | WTTES UTES Solid state Molten salts 0.4-0.9 kWh/m3·K (heat capacity) |
| Latent heat | 56-60, 30-85, 53.6(molten salt)4, 100-37017, 150-250 Wh/kg17, 56.521 |  |  |  |  | H – D | 300-3000 cycles | Low T PCM High T PCM |
| Thermochemical | 800-1200 200-350 180-310, 1.7MJ/kg4, 7014, | 1000-30003, 2500-29007, | 310-3607, |  |  | M - Year | >20 | Chemical looping Salt hydration Absorption |
| Hydrogen | 500 (200bar), 700 (300 bar), 0.8 kWh/L (350 bar) 4 1.9 kWh/L (700 bar)4, ~2540 (liquefied, -252.9℃) , 2.3 kWh/L (liquid) 4, 19018, | 3680 $/kW, 1570-3243€/kW2, 3117(2020)2, 1612(2030)2, | 312(2020)2, 161(2030)2, |  |  | D - M | 20 - 40 | *300 bar, pressured vessels #Salt cavern, 200 bar (max) & 70bar (min); cost doesn’t account for 1/3 of gas volume which cannot leave the cavern to keep minimal pressure |

Notes for Table 3:

1. Geo-mechanical energy storage, it pumps water underground to store energy in the elastic compression of rock formations1.

2. Hydrogen energy storage energy storage capacity cost can be as low as 0.5-5 $/kWh only applied for natural storage caverns, 10-15 $/kWh applied for compression steel tanks1.

3. The RTE of chemical energy storage is normally too low compared to others, mainly because multiple connected chemical processes, like for methane, methanation sub-process and CO2 capture process further reduces the RTE and increases the power cost for the overall power to methane to power system1.

4. Chemical storage systems also present relatively high power capacity cost due to the infrastructure required for the chemical processes, Achieving low charge and discharge power costs for chemical storage pathways thus depends on substantial future cost reductions for both electrolysis and fuel cells to $ $220/kW for each component1.

5. The resistive heat energy storage has higher discharge efficiency is due to combined heat and power cycle is considered1.

6. The diversity of storage options with different costs, efficiencies, design characteristics, and siting constraints may result in complementary technologies occupying specific subniches1.

7. S - seconds, H – Hours, D – Days, M – Months, Sea - seasonally,

8. Capital cost, as defined here, covers different components that vary by technology type. For batteries and capacitors, capital costs pertain to the procurement of the direct current (DC) energy storage unit and do not include PCS, BOP, or C&C costs. For electrochemical storage units, the capital cost reported within this report includes electrodes, electrolytes, and separators. For PSH, it includes waterways, reservoirs, pumps, and electrical generators. For CAES, it includes caverns, compressors, and generators. Total project costs, including installation cost. Power Conversion System ($/kW) of BESSs includes the cost for the inverter and packaging, as well as container and inverter controls. Fixed O&M includes all costs necessary to keep the storage system operational throughout the duration of its economic life that do not fluctuate based on energy usage, include labor, insurance, and taxes. Variable O&M includes all costs necessary to operate the storage system throughout the duration of its economic life and is normalized with respect to the annual discharge energy throughput. Variable O&M costs account for wear and tear of the system during operation19.

9. The capital component of a flow battery includes the electrolyte solution, membrane, and the hydraulic pumps necessary to push the solution from one tank to the other. The battery system can be composed of different design variants that can be stacked together to build systems that have larger capacities19.

10. The capital components of a conventional PSH facility include two water reservoirs, a waterway to connect them, and a power station that includes a pump and turbine.  The $/kW for electrical and mechanical equipment of PHS decreases with increasing power and is estimated to be $570/kW for a 4 MW system, $485/kW for a 48 MW system, and $245/kW for a 500 MW system. From 4.3 MW to 48 MW, the $/kW decreased by 15%, while from 48 MW to 500 MW, the drop is 50%19. The “mechanical and electrical machines,” was estimated to be 20 to 30 percent of total cost, which decreased linearly with increasing PSH power capacity.

11. The total capital cost of CAES plants includes equipment, construction, installation, engineering, and other costs necessary to build the grid-level storage system. When Pathfinder CAES in Utah was completed, the project would have a total of 1,500 MW/25,000 MWh for an E/P ratio of slightly over 1619. The established CAES plants, the McIntosh, Alabama, has a salt dome capacity of 18.9 MM ft3, and a pressure range of 4.5-7.6 MPa (650– 1,100 psia), corresponding to 55MW compression power, 3.37 GWh & 31 hours and 110MW discharge power. For Siemens SXT-800 CAES, 135 MW – 8.2 MPa or 160 MW – 15 MPa, the ramp rate is 20 percent of rated power per minute. The compressed air storage: 0.10 EUR/kWh in porous rock, 1.01 EUR/kWh in solution-mined salt caverns, 9.71 EUR/kWh in dry-mined salt caverns, 9.71 EUR/kWh in abandoned mines, and 29.55 EUR/kWh in rock caverns from excavation.

12. The hydrogen produced via electrolysis can then be transported and stored for subsequent electricity production using a hydrogen turbine or fuel cell or used as a transportation fuel. Hydrogen storage investment costs have been estimated as 0.002 EUR/kWh in porous rock, 0.02 EUR/kWh in solution-mined salt caverns, 0.14 EUR/kWh in dry-mined salt caverns, 0.14 EUR/kWh in abandoned mines, 0.25 EUR/kWh in geologic H2 storage, and 0.41 EUR/kWh in rock caverns from excavation12(2018).

13. Alkaline electrolysis has costs of 1000 EUR/kW [71] to 1400 EUR/kW [55] today, with a potential future cost of 500 EUR/kW [71]. Solid oxide electrolysis has a similar future cost potential of 590 EUR/kW by 2020 [55]. PEM electrolysers have a current cost of 2000 EUR/kW [71], with a near-term potential cost of 932 EUR/kW in 2025 [66] and 334 EUR/kW in 2050 [66]. Generic electrolysis costs are estimated at 1500 $/kW under business as usual assumptions [72] and 740 $/kW under optimistic assumptions [72].

Table 5. Electricity market services

| Function | Description | PHS | LAES | CAES | PTES-B | PTES-R | LCES | H2ES | VRFB | SES | PCMES | TCMES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Electricity arbitrage | Buying energy at a low price and selling at a high price on the wholesale or retail market | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | \ | \ | \ |
| Primary response | Ensuring network stability with sudden changes in frequency and voltage | \ | \ | \ | \ | \ | \ | ✓ | ✓ | \ | \ | \ |
| Secondary response | Ensuring predictability for unexpected differences between system load and power generation | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | \ | \ | \ |
| Tertiary response | Replacement of primary and secondary reaction with prolonged system overload | ✓ | ✓ | ✓ | \ | \ | ✓ | ✓ | \ | \ | \ | \ |
| Peak load shift | Ensuring adequate generation capacity during peak power demand periods | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | \ | \ | \ |
| Flexibility | Ensuing adequate power system flexibility when renewable added in | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Renewable firming & shift | Shifting renewable energy from oversupply to peak demand | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Seasonal electricity storage | Balancing the variability of seasonal electricity supply resulting from energy demand and supply | ✓ | \ | ✓ | \ | \ | \ | ✓ | \ | \ | \ | \ |
| Cold start/black start | Quick power plant recovery after network overload without additional external power supply | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | \ | \ | \ |
| Transmission network deferral | Deferring the reinforcement and improvement of network infrastructure | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | \ | \ | \ |
| Heat arbitrage | Purchase of electricity for heat production at a low price and sale of heat at a high price on the wholesale or retail market | \ | \ | \ | ✓ | ✓ | \ | \ | \ | ✓ | ✓ | ✓ |
| Seasonal heat storage | Balancing the variability of seasonal electricity supplies that are used to produce heat resulting from heat demand and supply | \ | \ | \ | ✓ | ✓ | \ | \ | \ | \ | ✓ | ✓ |

Notes:

- Energy time-shift service (sometimes called arbitrage) may provide a substantial part of the economic benefits/revenues (net utility avoided production costs) of a storage project. Energy time-shift value will progressively increase over time, as a function of the portfolio ofvariable energy resources and other factors, such as natural gas prices and the implementation of carbon emissions policies22.

References

1.	Sepulveda NA, Jenkins JD, Edington A, Mallapragada DS, Lester RK. The design space for long-duration energy storage in decarbonized power systems. Nat Energy. 2021;6(5):506-516. doi:10.1038/s41560-021-00796-8

2.	Mongird K, Viswanathan V, Alam J, Vartanian C, Sprenkle V, Northwest P. 2020 Grid Energy Storage Technology Cost and Performance Assessment.; 2020.

3.	The Royal Society. Large-Scale Electricity Storage.; 2023. royalsociety.org/electricity-storage

4.	Liang T, Li Y, Nie B, Ahmad A, Ding Y. Achieving a net-zero-carbon energy system in the UK by 2050 with liquid air energy storage. Energy Convers Manag. 2025;327(January):119524. doi:10.1016/j.enconman.2025.119524

5.	Vecchi A, Sciacovelli A. Long-duration thermo-mechanical energy storage – Present and future techno-economic competitiveness. Appl Energy. 2023;334(December 2022):120628. doi:10.1016/j.apenergy.2022.120628

6.	Jacob R, Hoffmann M, Weinand JM, Linßen J, Stolten D, Müller M. The future role of thermal energy storage in 100% renewable electricity systems. Renew Sustain Energy Transit. 2023;4(July). doi:10.1016/j.rset.2023.100059

7.	Vecchi A, Knobloch K, Liang T, et al. Carnot Battery development: A review on system performance, applications and commercial state-of-the-art. J Energy Storage. 2022;55(PD):105782. doi:10.1016/j.est.2022.105782

8.	Mctigue J, Farres-Antunez P, White A. Crescent Dunes Solar Energy Facility, USA Pumped thermal energy storage: thermodynamics and economics. Published online 2019.

9.	Sun L, Tang B, Xie Y. Performance assessment of two compressed and liquid carbon dioxide energy storage systems: Thermodynamic, exergoeconomic analysis and multi-objective optimization. Energy. 2022;256:124648. doi:10.1016/j.energy.2022.124648

10.	Guccione S, Guedez R. Techno-economic analysis of power-to-heat-to-power plants: Mapping optimal combinations of thermal energy storage and power cycles. Energy. 2024;312(February):133500. doi:10.1016/j.energy.2024.133500

11.	Belik S. Techno-economic evaluation of a Brayton battery configuration with power-to-heat extension. J Energy Storage. 2023;68(June 2022):107416. doi:10.1016/j.est.2023.107416

12.	McPherson M, Johnson N, Strubegger M. The role of electricity storage and hydrogen technologies in enabling global low-carbon energy transitions. Appl Energy. 2018;216(December 2017):649-661. doi:10.1016/j.apenergy.2018.02.110

13.	Smith CL. Large-Scale Electricity Storage Policy Briefing.; 2023. https://royalsociety.org/-/media/policy/projects/large-scale-electricity-storage/Large-scale-electricity-storage-policy-briefing.pdf

14.	Shamsi SSM, Barberis S, Maccarini S, Traverso A. Large scale energy storage systems based on carbon dioxide thermal cycles: A critical review. Renew Sustain Energy Rev. 2024;192(June 2023):114245. doi:10.1016/j.rser.2023.114245

15.	Vallese L, Javadi H, Badenes B, et al. A comprehensive review of thermal energy storage technologies and their applications: Creation of a database. Renew Sustain Energy Rev. 2026;225(June 2025). doi:10.1016/j.rser.2025.116133

16.	Zakeri B, Syri S. Electrical energy storage systems: A comparative life cycle cost analysis. Renew Sustain Energy Rev. 2015;42:569-596. doi:10.1016/j.rser.2014.10.011

17.	Koohi-Fayegh S, Rosen MA. A review of energy storage types, applications and recent developments. J Energy Storage. 2020;27:101047. doi:10.1016/j.est.2019.101047

18.	Blanco H, Faaij A. A review at the role of storage in energy systems with a focus on Power to Gas and long-term storage. Renew Sustain Energy Rev. 2018;81(May 2017):1049-1086. doi:10.1016/j.rser.2017.07.062

19.	Mongird K, Viswanathan V, Balducci P, et al. Energy Storage Technology and Cost Characterization Report | Department of Energy. US Dep Energy. 2019;(July):1-120. https://www.energy.gov/eere/water/downloads/energy-storage-technology-and-cost-characterization-report

20.	McTigue J, Farres-Antunez P, Ellingwood K, Neises T, White A. Pumped thermal electricity storage with supercritical CO2 cycles and solar heat input. Vii Int Young Res Conf – Physics, Technol Innov. 2020;2313(September):190024. doi:10.1063/5.0032337

21.	Zhang Y, Johansson P, Kalagasidis AS. Techno-economic assessment of thermal energy storage technologies for demand-side management in low-temperature individual heating systems. Energy. 2021;236. doi:10.1016/j.energy.2021.121496

22.	Helman U, Kaun B, Stekli J. Development of Long-Duration Energy Storage Projects in Electric Power Systems in the United States: A Survey of Factors Which Are Shaping the Market. Front Energy Res. 2020;8(November):1-15. doi:10.3389/fenrg.2020.539752

References for Table 2

[1] International Electrochemical Commission (IEC). White paper report: electrical energy storage. 2011. http://www.iec.ch/whitepaper/pdf/iecWP-energystorage-LR-en.pdf. Accessed in May 2019.

[2] A. Sabihuddin, A.E. Kiprakis, M. Mueller, A numerical and graphical review of energy storage technologies, Energies 8 (2015) 172–216.

[3] International Energy Agency (IEA). Technology roadmap: energy storage. 2014. https://www.iea.org/publications/freepublications/publication/TechnologyRoadmapEnergystorage.pdf. Accessed in May 2019.

[4] H. Chen, T.N. Cong, W. Yang, C. Tan, Y. Li, Y. Ding, Progress in electrical energy storage system: a critical review, Progress Nat. Sci. 19 (3) (2009) 291–312.

[5] Andreas Olympios et al 2021 Prog. Energy in press https://doi.org/10.1088/2516-1083/abdbb

[6] F. Meishner, D.U. Sauer, Wayside energy recovery systems in DC urban railway grids, eTransportation 1 (2019) 100001.

[7] International Renewable Energy Agency (IRENA). Innovation outlook: Thermal energy storage. 2020. https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2020/Nov/IRENA_Innovation_Outlook_TES_2020.pdf . Accessed in February 2021.

[8] Strategic Analysis, “Final report: hydrogen storage system cost analysis,” 2016.

[9] A. Le Duigou, A.-G. Bader, J.-C. Lanoix, and L. Nadau, “Relevance and costs of large scale underground hydrogen storage in France,” Int. J. Hydrogen Energy, vol. 42, no. 36, pp. 22987–23003, Sep. 2017.

[10] Bañares-Alcántara et al: Analysis of Islanded Ammonia-based Energy Storage Systems, University of Oxford, 09/2015